Introduction: Particle size analysis is the determination of percent sand, silt and clay components in a soil. The Bouyoucos Hydrometer method adapted from Gee and Bauder (1986) is used to determine the soil texture of the soil sample. This method follows USDA soil classifications for particle sizes.

Materials:

- Standard hydrometer, American Society for Testing and Materials (ASTM) no. 152 H, with Bouyoucos scale in g/L

- Plunger or rubber stoppers for 1000-mL sedimentation cylinders

- Sedimentation cylinders with 1000-mL mark

- Sodium-hexametaphosphate (HMP) solution (50 g/L)

- Balance

- Stop watch/ timer

- Distilled water

- Thermometer

HMP Solution Preparation

Measure 50 g sodium hexametaphosphate on a balance. Combine sodium hexametaphosphate and distilled water for a total volume of 1000 mL. Stir until dissolved.

Soil Dispersion

Grind dry soil to break up chunks and loosen clods. Use 2mm mesh sieve to remove gravel. Weigh 40 g of dry soil and put it into a 600-mL beaker. Add 250 mL distilled water and 100 ml HMP. Soak over night. Transfer soil/ HMP mixture to dispersing cup and mix for 5 min. Pour mixture into sedimentation cylinder and add distilled water to reach 1 L.

Calibration with a Blank

In a sedimentation cylinder, add 100 mL HMP to 900 mL room temperature distilled water to make 1L total volume solution. Mix with plunger and record temperature. Gently, lower the hydrometer into the cylinder. Read and record where the upper edge of the meniscus surrounds the stem of the hydrometer. Periodically throughout the test, read and record this measurement (RL ) for solution viscosity and soil concentration analysis corrections.

Hydrometer Measurements

Allow samples to reach ambient temperature and record temperature. Insert plunger into cylinder and mix thoroughly. Use upward strokes to remove sediment from bottom and finish with five

or six smooth strokes. An alternative to plunging is to stopper the cylinder and shake it end over end for 1 min. When mixing is complete, gently place hydrometer into the suspension. Take a reading at 30 s and take a reading at 60 s. Remove, rinse and dry hydrometer. Reinsert hydrometer 10 s before each reading. Take additional readings at 90 min and again at 1440 min. Record these as R. Also record RL from the calibration blank and the temperature at each time.

CALCULATIONS

Derivations and sources for these equations can be found in Gee and Bauder (1986).

Preliminary Calculations

The calculated value for B can be used for all time intervals if temperature of samples was kept relatively constant throughout. B provides corrections for the density and viscosity variations of the HMP solution.

B = 30η / [g(ρs - ρl)]

Where: η = fluid HMP solution viscosity in poise, g cm-1s-1

g = gravitational constant, cm/s2, Constant 980 cm/s2

ρs = soil particle density, g/cm3, assume standard 2.65 g/cm3

ρl = HMP solution density, g/cm3

ρl= ρo ( 1 + 0.630 CS )

Where: ρl = HMP solution density at temperature t, g/ cm3

ρo = water density at temperature t, g/ cm3

At temp 20° C = 0.998 g/cm3, at temp 25° C = 0.997 g/ cm3

CS = concentration of HMP, g/ cm3, 0.05 g/cm3

η = ηo (1+ 4.25 CS)

Where: η = HMP solution viscosity at temperature t in poise, g cm-1s-1

ηo = water viscosity at temperature t in poise, g cm-1s-1

At temp 20° C = 0.01 g cm-1s-1, at temp 25° C = 0.0089 g cm-1s-1

CS = concentration of HMP, g/ cm3, 0.05 g/cm3

Particle Diameter (X) Calculation for Each Time Interval

These calculations are used to determine particle diameter in solution at each time (t) interval.

X = θ(t)-1/2

Where: X = mean particle diameter in suspension, μm at time t

θ = sedimentation parameter, μm min1/2

t = time in minutes

θ = 1000(Bh′)1/2

Where: θ = sedimentation parameter, μm min1/2

h′ = effective hydrometer depth, cm

h′ = -0.164 R +16.3

Where: h′ = effective hydrometer depth, cm

R = uncorrected hydrometer reading, g/L

Summation Percentage (P) for each Time Interval

P = C/Co × 100

Where: P = summation of percentage for given time interval

C = concentration of soil in suspension, g/ L

Co = dry weight of soil sample used for test.

C = R – RL

Where: C = concentration of soil in suspension, g/ L

R = uncorrected hydrometer reading, g/ L

RL = hydrometer reading of blank solution, g/ L

Clay Fraction Determination

Use hydrometer readings, both R and RL, at 1.5 h and 24 h to determine effective particle diameter (X) and summation percentage (P) for 1.5 h and 24h readings.

Calculate percent clays (P2μm) with equation: P2μm = m ln (2 / X24) + P24

Where: X24 = mean particle diameter in suspension at 24 h.

P24 = summation of percentage at 24h.

m = (P1.5  – P24) / ln (X1.5 / X24)= slope of the summation percentage curve between X at 1.5 h and X at 24 h

X1.5  = mean particle diameter in suspension at 1.5 h.

P1.5  = summation percentage at 1.5 h.

Sand Fraction Determination.

Use hydrometer reading, both R and RL, at 30 s and 60 s to calculate summation percentages for 30 s and 1 m. Calculate P50μm with equation: P50μm = m ln (50 / X60) + P60

Where: X60 = mean particle diameter in suspension at 60 s.

P60 = summation of percentage at 60 s.

m = (P30– P60) / ln (X30 / X60)= slope of the summation percentage curve between X at 30 s and X at 60 s (1 min).

X30 = mean particle diameter in suspension at 30 s.

P30 = summation percentage at 30 s.

Subtract computed P50 μm value from 100 to obtain sand percentage

Silt Fraction Determination

Use the equation: % silt = 100 – (% sand + % clay)

REFERENCES

Garretson, C. (1999a). Soil Water Content Determination. SOP METH001.00. Environmental Monitoring Branch, DPR. available on-line: http://www.cdpr.ca.gov/docs/empm/pubs/sop.htm.

Garretson, C. (1999c). Soil Bulk Density Determination. SOP FSSO001.00. Environmental Monitoring Branch, DPR. available on-line: http://www.cdpr.ca.gov/docs/empm/pubs/sop.htm.

Gee, G.W. and J.W. Bauder. 1986. Particle-size Analysis in Klute, Arnold, ed.,Methods of Soil Analysis Part 1, Physical and Mineral Methods Second Edition. American Society of Agronomy- Soil Science Society of America, 677 South Segoe Road, Madison, WI 53711, USA.

Material Safety Data Sheet for Sodium Hexametaphosphate, Spectrum, Gardena, Ca. Available online at http://www.spectrumchemical.com/MSDS/S4070.pdf

Material Safety Data Sheet for 1-Pentanol, Available online at http://msds.ehs.cornell.edu/msds/msdsdod/a435/m217192.htm.